Automated stock/level monitoring

Hardware:

- JSN-SR04T Waterproof Ultrasonic Distance Sensor (depends on the number of sensors to be installed)

- Bi-Directional Logic Level Converter (depends on the number of sensors to be installed)

- Pin header (optional)

- Raspberry Pi 4 + microSD card

- Jumper wires

- Soldering tools

Installation suggestion:

|  | Raspberry Pi | Bi-Directional Logic Level Converter | JSN-SR04T Waterproof Ultrasonic Distance Sensor |
| --- | --- | --- | --- |
| Quantity | 1 | 1 | 1 |
|  |  |  | 2 |
|  |  | 2 | 3 |
|  |  |  | 4 |

Circuit diagram:

Software:

- Clone the code from GitHub repository:  Open a Terminal (Click the ‘Terminal’ icon in the taskbar) and type in the following.  Wait until the cloning process completes.

git clone https://github.com/jyl12/level-monitoring.git

Or unzip the folder to the following path (Click the ‘File Manager’ icon in the taskbar)

/home/pi

- Install necessary software libraries:  Type the following in the Terminal (continue from Step 1 or start a new Terminal).  Wait until the installation process completes. Type Y for yes and continue if prompted.

cd level-monitoring

sh install.sh

- Run the code:  Type the following in the Terminal to start.

python3 /userInterface/app.py

If it is a new Terminal, type the following.

python3 /level-monitoring/userInterface/app.py

Once it starts, the web interface should be accessible at http://localhost:5000 or from another device at http://<raspberrypi-IP>:5000 (replaces <raspberrypi-IP> with the IP address of the Raspberry Pi in the network).  All you need to find out the IP address of your Raspberry Pi is to run either one of the following commands in the terminal.

hostname -I

ifconfig

Email (optional):

In sendemail.py, please modify #Email Variables accordingly to enable the email function.

#Email Variables SMTP_SERVER = 'smtp.gmail.com' #Email Server (don't change!) SMTP_PORT = 587 #Server Port (don't change!) GMAIL_USERNAME = 'shoestringtestzy@gmail.com' #change this to match your gmail account GMAIL_PASSWORD = 'Pass1234!' #change this to match your gmail password

In analysis.py, set ENABLE_EMAIL = 1 to enable the email and change USER_EMAIL to relevant emails.

ENABLE_EMAIL = 0 #1 to enable email sending USER_EMAIL = ['jyl49@cam.ac.uk'] # ['abc@xyz.com','efg@xyz.com']

Run Program on Startup (Optional):

- Create a .desktop File.  Open a Terminal and execute the following commands to create an autostart directory (if one does not already exist) and edit a .desktop file.

mkdir /home/pi/.config/autostart

nano /home/pi/.config/autostart/startapp.desktop

- Copy in the following text into the startapp.desktop file. Feel free to change the Name and Exec variables to your particular application.

[Desktop Entry] Type=Application Name=App Exec=@/usr/bin/python3 /home/pi/level-monitoring/userInterface/app.py

- Save and exit with ctrl + x, followed by y when prompted to save, and then enter. Reboot with the following command.

sudo reboot

Please refer Method 2:autostart of the following hyperlink for detailed explanation.

https://learn.sparkfun.com/tutorials/how-to-run-a-raspberry-pi-program-on-startup/all#method-2-autostart

Troubleshoot:

- OPC UA port: 4840. When connecting UaExpert to Raspberry Pi, the following logs might appear in the terminal console of Raspberry Pi and UaExpert.

Solution: Can be ignored. Error message is caused by missing PublishResult.Results acknowledgements on freeopcua. Please refer the following links for more information. https://github.com/FreeOpcUa/opcua-asyncio/issues/438 https://github.com/FreeOpcUa/python-opcua/issues/803